- What is UP and DOWN Counter? Explain both with ladder diagram, address and Instruction bits?

- What is OSR and OSF Counter? Explain both ?

- What are the different types of Comparison Instructions? Explain any two with instruction diagram & address?

- What is Move function block, Explain with ladder diagram?

- Write the PLC Ladder programme for Direct ON Line (DOL) Starter with addressing and names?

- Write the Ladder program for Single Way Traffic Signal Control

  - Start the single way traffic signal cycle by pressing Start Push Button

  - Red Light gets ON,Timer 1 for 30 Seconds to Stop.

  - Yellow Lights ON, Timer 2 ON for 5 Seconds To Ready.

  - Green Lights ON, Timer 3 ON for 25 Seconds to GO.

  - Continue the cycles as per the above sequence till we press the Emergency OFF or Stop Push Button.

- What are the different types of SCADA Systems ? Explain all ?

- Explain SCADA tags ?

- Write PLC Programme for below Diagram?

  - Motor ON & OFF using Start and Stop push button (DOL starter)

  - Motor ON - Low Level Switch Trip

  - Motor OFF - High Level Switch Trip

- Explain Latch and Unlatch Blocks using ladder ?

- Write the PLC Programming Motors logic for the following problem.

  - There are three motors (M1, M2 & M3), having a separate start and stop push button.

  - M1 must be running before starting of M2 and M3 motor.

  - M2 and M3 can start and stop without affecting M1 operation.

  - Only two motors can run at the time. Starting of the third motor will shut down all the outputs immediately irrespective of its input.

- Write the PLC Programme,

  - There are three mixing device motor in processing line A, B, C.

  - After the process begins mixer A is to start

  - After 15 seconds of starting mixer A  next mixer B is to start

  - After 10 seconds of starting mixer B next mixer C to start.

  - The entire mixer to stop after pressing master switch.

- Explain SCADA briefly with meaning of S, C and DA? What are the types/generation of SCADA?